CAD DATA LABELLING

PROJECT

A comprehensive technical framework for 3D CAD model annotation,

machine learning data preparation, and feature recognition training

Prepared for:

Isidor AI

Date: January 2026

# Table of Contents

1. Executive summary

2. Understanding CAD File Formats and Data Structures

3. Files and Data Required for Labelling

4. Project Structure and Data Pipeline

5. Feature Labelling Taxonomy

6. Labelled Data Format Specification

7. Real-World Example: Roller Shaft Subassembly

8. Complexity Challenges and Labelling Design Decisions

9. Data Preprocessing and Simplification

10. CAD Labelling Approaches and Industry Technologies

11. Labeller Training Programme

12. Project Scope, Timeline, and Scaling

13. Future Scope and Strategic Opportunities

14. Implications and Strategic Considerations

15. Conclusion and Recommended Next Steps

16. Bibliography and References used

# 1. Executive Summary

This proposal presents a comprehensive technical framework for developing a CAD (Computer-Aided Design) data labelling initiative tailored to Isidor's mission of transforming raw, unstructured proprietary enterprise data into reinforcement-learning-ready curriculum for AI model training. The project addresses the critical industry need for high-quality, annotated 3D CAD datasets that can power next-generation machine learning models in manufacturing automation, design synthesis, and engineering applications.

CAD data represents one of the most valuable yet underutilised sources of enterprise data. The global data annotation and labelling market is projected to grow from USD 0.8 billion in 2022 to USD 3.6 billion by 2027 (CAGR 33.2%), with CAD-specific labelling emerging as a high-value niche. Unlike image or text annotation, CAD labelling requires specialised domain expertise in engineering, manufacturing processes, and geometric reasoning, creating significant barriers to entry and competitive differentiation.

## 1.1 Key technical challenges addressed

- Boundary Representation (B-Rep) complexity: CAD models store geometry as faces, edges, and vertices with complex topological relationships that must be preserved during labelling to enable downstream graph neural network (GNN) training.

- Feature intersection handling: Unlike simple object detection, machining features (holes, pockets, slots) often intersect, creating ambiguous boundaries that require expert judgment and consistent labelling protocols.

- Multi-Resolution requirements: Different ML architectures require different input formats, voxels for 3D CNNs, point clouds for PointNet++, face adjacency graphs for GNNs, necessitating flexible preprocessing pipelines.

- Scarcity of annotated data: While millions of CAD models exist in enterprise PLM systems, professionally labelled datasets remain scarce. The MFCAD++ dataset (59,665 models) represents the current state-of-the-art, highlighting the opportunity for data labelling companies.

- Domain expertise requirements: Labellers must understand manufacturing processes, GD&T (Geometric Dimensioning and Tolerancing), and engineering conventions to produce accurate annotations.

# 2. Understanding CAD file formats and data structures

A deep understanding of CAD file formats is essential for designing an effective labelling pipeline. CAD files are fundamentally different from images or text because they contain rich geometric, topological, and semantic information that must be preserved and leveraged during annotation.

## 2.1 Boundary representation (B-Rep) fundamentals

B-Rep is the de facto standard for representing solid models in professional CAD systems (CATIA, SolidWorks, NX, Creo). Understanding B-Rep is critical because it defines what information is available for labelling and how features can be identified:

- Vertices (0D entities): Points in 3D space where edges meet. Each vertex has (x, y, z) coordinates. In a simple cube, there are 8 vertices. Vertices define the corners of features like rectangular pockets or the centres of holes.

- Edges (1D entities): Curves connecting vertices. Edges can be lines (straight), arcs (circular segments), or NURBS curves (free-form). Each edge has a start vertex, end vertex, and curve geometry. Edges define the boundaries between faces.

- Faces (2D entities): Bounded surfaces forming the outer shell of the solid. Faces can be planar, cylindrical, conical, spherical, toroidal, or B-spline. Each face has a surface geometry, boundary loops of edges, and a normal vector pointing outward. Faces are the primary unit of labelling.

- Coedges (Oriented Edges): Each edge appears twice in B-Rep, once for each adjacent face, with opposite orientations. Coedges maintain the topological consistency of the model and enable traversal of adjacent faces.

## 2.2 Native vs neutral file formats

CAD files fall into two categories, each with implications for the labelling pipeline:

### Native formats (Proprietary)

- CATIA (.CATPart, .CATProduct): Dassault Systems' format used extensively in aerospace and automotive. Contains full parametric history, constraints, and feature tree. Requires CATIA license to read.

- SolidWorks (.SLDPRT, .SLDASM): Popular in mid-market manufacturing. Contains feature tree, configurations, and design intent. Requires SolidWorks API for extraction.

- Siemens NX (.prt): Used in automotive and industrial machinery. Contains PMI, CAM data, and simulation results.

### Neutral formats (Interoperable)

- STEP AP242 (.stp, .step): ISO 10303 standard. The recommended format for CAD labelling. Preserves complete B-Rep geometry, PMI (Product Manufacturing Information), GD&T, assembly structure, and material assignments. Supported by all major CAD systems.

- Parasolid (.x_t, .x_b): Siemens' geometric kernel format. Excellent geometric fidelity for solid models. Used internally by SolidWorks, NX, and Solid Edge.

- DXF (.dxf): AutoCAD's Drawing Exchange Format. Primarily 2D but supports 3D entities. As Chen et al. (2024) demonstrate, DXF files contain structured geometric data (lines, circles, arcs, polylines) that can be automatically parsed and labelled without neural network approaches.

## 2.3 File format comparison for data labelling

| Format | B-Rep | PMI/GD&T | Assembly | Colour | Labelling Use |
| --- | --- | --- | --- | --- | --- |
| STEP AP242 | ✓ Full | ✓ Full | ✓ Full | ✓ | Excellent - Primary |
| Parasolid | ✓ Full | ✗ | ✓ Full | ✓ | Excellent |
| DXF | 2D/Basic 3D | Dimensions | ✓ Layers | ✓ | Good - 2D Drawings |
| STL | ✗ Mesh only | ✗ | ✗ | ✗ | Limited - Voxel input |

Table 1: File format capabilities for CAD data labelling

# 3. Files and Data Required for Labelling

A comprehensive CAD data labelling project requires multiple file types and metadata sources to create complete, high-quality training datasets.

## 3.1 Primary 3D model files

- STEP/Parasolid file (Required): The authoritative geometric representation. Must be a valid, watertight solid. File should be exported with maximum precision settings (typically 0.001mm chord tolerance).

- Tessellated mesh (Generated): An STL or OBJ mesh derived from the B-Rep with controlled resolution. Recommended settings: maximum edge length 1mm, angular deviation 15°, chord height 0.1mm.

- Multi-View rendered images (Generated): 2D renders of the 3D model from standardised viewpoints (front, back, left, right, top, bottom, isometric). Resolution: minimum 512×512 pixels. Used for labeller reference and for training vision-language models.

## 3.2 Metadata requirements

- Unique identifier: A globally unique ID for each model (e.g., UUID v4). This ID links all associated files, labels, and audit trails.

- Part classification: Industry sector (aerospace, automotive, medical, consumer), part family (bracket, housing, shaft, gear), functional role (structural, cosmetic, wear component).

- Geometric properties (Computed): Bounding box dimensions, total volume (mm³), total surface area (mm²), centre of mass coordinates, moments of inertia.

- Manufacturing context: Intended manufacturing process (CNC milling, turning, casting, additive), material class (aluminium, steel, titanium, polymer), tolerance grade (ISO IT5-IT14).

# 4. Project Structure and Data Pipeline

## 4.1 End-to-End data pipeline architecture

The CAD labelling pipeline transforms raw client CAD files into ML-ready annotated datasets through six stages. Each stage includes detailed processes, quality gates, and output specifications:

### Stage 1: Data Ingestion and Validation

- File collection: Receive CAD files via secure upload (SFTP, S3 with client-side encryption, or API). Support batch uploads of 100-10,000 files.

- Format cetection: Automatically identify file format from extension and magic bytes. Following Chen et al.'s methodology, the program determines file type to call the appropriate inference program.

- Geometric validation: Run automated checks: solid is watertight (Euler characteristic = 2 for simple solids), no self-intersections, consistent face orientations.

### Stage 2: Preprocessing for ML Representations

- Object-Oriented conversion: As per Chen et al.'s approach, convert unstructured CAD files into structured data with object-oriented representation. Generate a set of entities consisting of m lines, n circles/arcs, and p polylines.

- Voxelisation: Convert solid to 3D occupancy grid using binvox or custom ray-casting. Standard resolutions: 32³ (fast preview), 64³ (training default), 128³ (high-fidelity).

- Face Adjacency Graph (AAG) Construction: Build graph where nodes=faces, edges=shared boundaries. Store surface_type, area, centroid, normal vector, principal curvatures for each node.

### Stage 3: Labelling Execution

- Face-Level labelling: Assign one feature class to each face in the B-Rep model. Example: Face #23 → 'through_hole_cylindrical'.

- Automatic feature inference: Following Chen et al.'s methodology, identify specific geometrical characteristics as anchors, then use domain knowledge to infer and auto-label key entities.

Stage 4: Quality Assurance

- Automated checks: Verify all faces labelled (no orphans), labels are valid taxonomy entries, feature instances are connected

- Sampling-based review: Senior reviewer examines 10-20% of completed models

- Inter-annotator agreement (IAA): Assign 5-10% of models to multiple labellers, target Cohen's Kappa κ > 0.85

Stage 5: Feature Instance Grouping

- Group labelled faces into feature instances (e.g., Faces #23, #24, #25 form 'Hole_Instance_1')

- Relationship annotation: Record feature relationships ('Hole_1 is coaxial with Hole_2')

- Confidence scoring: Record labeller confidence (1-5 scale) for each label

Stage 6: Export and Delivery

- Dataset packaging: Assemble final dataset with directory structure (/step/, /mesh/, /voxel/, /graphs/, /labels/, /splits/)

- Label format: Export in standardised JSON schema with face-level and instance-level labels

- Split generation: Create train/val/test splits (70/15/15), stratified by feature type and complexity

- Documentation: Include taxonomy definition, labelling guidelines, and data card

# 5. Feature Labelling Taxonomy

The labelling taxonomy defines the classification schema for CAD features. Based on industry standards (ISO 10303, STEP-NC) and existing research datasets (MFCAD++, FeatureNet), we propose a hierarchical 25-class taxonomy for machining features:

| ID | Feature Name | Description | Faces |
| --- | --- | --- | --- |
| 0 | stock | Base material not part of any feature | 1-6 |
| 1 | through_hole | Cylindrical hole passing completely through | 1-2 |
| 2 | blind_hole | Cylindrical hole with flat or conical bottom | 2-3 |
| 3 | countersink | Conical recess at hole entry | 1 |
| 4 | counterbore | Cylindrical recess at hole entry | 2-3 |
| 5 | rectangular_pocket | Rectangular depression with flat bottom | 5-9 |
| 6 | rectangular_through_slot | Rectangular channel passing through | 3-5 |
| 7 | rectangular_blind_slot | Rectangular channel with closed end | 4-6 |
| 8 | rectangular_passage | Rectangular opening through material | 4 |
| 9 | rectangular_through_step | L-shaped removal at edge (open both ends) | 2-3 |
| 10 | rectangular_blind_step | L-shaped removal at edge (one closed end) | 3-4 |
| 11 | triangular_pocket | Triangular depression | 4-6 |
| 12 | triangular_passage | Triangular opening through | 3 |
| 13 | triangular_through_slot | Triangular channel passing through | 3-4 |
| 14 | triangular_blind_step | Triangular step with closed end | 3-4 |
| 15 | two_sided_through_step | Step open on two opposite sides | 2 |
| 16 | slanted_through_step | Angled step passing through | 2-3 |
| 17 | chamfer | Angled edge transition | 1 |
| 18 | six_sided_pocket | Hexagonal depression | 7-9 |
| 19 | six_sided_passage | Hexagonal opening through | 6 |
| 20 | circular_through_slot | Circular arc channel through | 2-4 |
| 21 | circular_end_pocket | Pocket with circular/rounded ends | 3-5 |
| 22 | circular_end_blind_slot | Rounded-end slot with closed end | 4-6 |
| 23 | horizontal_circular_end_blind_slot | Horizontal rounded slot, closed | 4-6 |
| 24 | round_fillet | Rounded edge transition (convex or concave) | 1 |

Source: MFCAD++ Dataset (59,665 CAD models, 70/15/15 train/val/test split)

Table 2: Sample machining feature taxonomy (25 classes total in MFCAD++ standard)

# 6. Labelled Data Format Specification

Each labelled CAD model produces a JSON file with a standardised schema. This format is compatible with state-of-the-art CAD feature recognition frameworks (Hierarchical CADNet, BRepNet, AAGNet).

## 6.1 JSON schema overview

The structured form for converting imported CAD files to export data follows Chen et al.'s methodology, with extensions for ML training:

| Class | Property | Format | Unit |
| --- | --- | --- | --- |
| Line | Layer, Start point, End point | String, [Float, Float] | --, mm |
| Circle | Layer, Radius, Central point | String, Float, [Float, Float] | --, mm, mm |
| Arc | Radius, Central point, Start/End angle | Float, [Float, Float], Float | mm, mm, radian |
| Polyline | Layer, All vertices | String, [[Float, Float], ...] | --, mm |

Table 3: Structured form for converting DXF files (adapted from Chen et al., 2024)

# 7. Real-World Example: Roller Shaft Subassembly

To demonstrate the practical application of the CAD data labelling framework, the following complete example is presented using a roller shaft subassembly engineering drawing. This example shows how a real engineering drawing would be translated into structured JSON format for machine learning applications.

## 7.1 Engineering drawing: Roller shaft subassembly

The following engineering drawing shows a roller shaft subassembly designed at Imperial College London, Department of Mechanical Engineering. The drawing includes an isometric view, section view, and a complete bill of materials (BOM) with 12 components.

Figure 1: Roller Shaft Subassembly Engineering Drawing (Imperial College London)

## 7.2 Bill of Materials (BOM) extraction

From the drawing, we extract the following components:

| ITEM NO. | PART NUMBER | DESCRIPTION | QTY. |
| --- | --- | --- | --- |
| 1 | BSA-1 | MAIN SHAFT WITH ROLLER | 1 |
| 2 | BS 3673-4 - S010M | EXTERNAL CIRCLIP 10MM | 2 |
| 3 | BS 3673-4 - S012M | EXTERNAL CIRCLIP 12MM | 1 |
| 4 | BS 3673-4 - S014M | EXTERNAL CIRCLIP 14MM | 1 |
| 5 | S112B30H14L12.0S1 | 112 TEETH MOD 1 SPUR GEAR | 1 |
| 6 | BSA-2 | KEY 5MM * 5MM | 1 |
| 7 | BSA-3 | ROLLER 40MM OUTER DIA | 1 |
| 8 | KFL-000 | FLANGE BEARING HOUSING 10MM | 1 |
| 9 | A-KSTM-10 | PILLOW BLOCK HOUSING 10MM | 1 |
| 10 | BSA-4 | PLAIN BUSH 14MM INNER DIA | 1 |
| 11 | BSA-5 | FLANGE 16MM INNER DIA | 1 |
| 12 | 18x1.8-A-ISO 3601-1 | O-RING 18MM | 1 |

Table 4: Extracted Bill of Materials from Roller Shaft Subassembly

## 7.3 Complete JSON label format for roller shaft subassembly

The following JSON structure demonstrates how the roller shaft subassembly would be labelled for ML training. This includes assembly-level metadata, component relationships, mate definitions, and feature annotations:

{

"assembly_metadata": {

"assembly_id": "isidor_asm_roller-shaft-001",

"title": "ROLLER SHAFT SUBASSEMBLY",

"drawing_number": "Roller shaft subassembly drawing",

"institution": "Imperial College London",

"department": "Mechanical Engineering",

"drawn_by": "YI LU",

"date": "2023/2/16",

"scale": "2:3",

"sheet_size": "A3",

"units": "millimetres",

"tolerances": {

"linear_X": 0.5,

"linear_XX": 0.1,

"linear_XXX": 0.02,

"angular": 1.0,

"surface_finish": "Ra 6.3"

},

"projection": "third_angle"

},

"bill_of_materials": [

{

"item_no": 1,

"part_number": "BSA-1",

"description": "MAIN SHAFT WITH ROLLER",

"quantity": 1,

"part_type": "custom_machined",

"material": "steel",

"features": [

"shaft",

"keyway",

"circlip_grooves"

]

},

{

"item_no": 2,

"part_number": "BS 3673-4 - S010M",

"description": "EXTERNAL CIRCLIP 10MM",

"quantity": 2,

"part_type": "standard_component",

"standard": "BS 3673-4"

},

{

"item_no": 5,

"part_number": "S112B30H14L12.0S1",

"description": "112 TEETH MOD 1 SPUR GEAR",

"quantity": 1,

"part_type": "standard_component",

"gear_parameters": {

"teeth": 112,

"module": 1,

"bore_diameter": 14,

"face_width": 12

}

},

{

"item_no": 6,

"part_number": "BSA-2",

"description": "KEY 5MM * 5MM",

"quantity": 1,

"part_type": "standard_component",

"standard": "DIN 6885"

},

{

"item_no": 7,

"part_number": "BSA-3",

"description": "ROLLER 40MM OUTER DIA",

"quantity": 1,

"part_type": "custom_machined"

},

{

"item_no": 8,

"part_number": "KFL-000",

"description": "FLANGE BEARING HOUSING 10MM",

"quantity": 1,

"part_type": "standard_component"

},

{

"item_no": 9,

"part_number": "A-KSTM-10",

"description": "PILLOW BLOCK HOUSING 10MM",

"quantity": 1,

"part_type": "standard_component"

},

{

"item_no": 10,

"part_number": "BSA-4",

"description": "PLAIN BUSH 14MM INNER DIA",

"quantity": 1,

"part_type": "custom_machined"

},

{

"item_no": 11,

"part_number": "BSA-5",

"description": "FLANGE 16MM INNER DIA",

"quantity": 1,

"part_type": "custom_machined"

},

{

"item_no": 12,

"part_number": "18x1.8-A-ISO 3601-1",

"description": "O-RING 18MM",

"quantity": 1,

"part_type": "standard_component",

"standard": "ISO 3601-1"

}

],

"assembly_mates": [

{

"mate_id": "mate_001",

"mate_type": "cylindrical",

"dof": 2,

"part_a": {

"part_id": "BSA-1",

"reference_entity": {

"entity_type": "face",

"surface_type": "cylinder",

"feature": "shaft_diameter_10mm",

"mcf_location": "axis_centroid"

}

},

"part_b": {

"part_id": "KFL-000",

"reference_entity": {

"entity_type": "face",

"surface_type": "cylinder",

"feature": "bearing_bore",

"mcf_location": "axis_centroid"

}

},

"constraint_params": {

"axes_aligned": true,

"clearance_fit": "H7/g6"

}

},

{

"mate_id": "mate_002",

"mate_type": "fastened",

"dof": 0,

"part_a": {

"part_id": "BSA-1",

"reference_entity": {

"entity_type": "face",

"surface_type": "plane",

"feature": "keyway_bottom"

}

},

"part_b": {

"part_id": "BSA-2",

"reference_entity": {

"entity_type": "face",

"surface_type": "plane",

"feature": "key_bottom"

}

}

},

{

"mate_id": "mate_003",

"mate_type": "fastened",

"dof": 0,

"part_a": {

"part_id": "S112B30H14L12.0S1",

"reference_entity": {

"entity_type": "face",

"surface_type": "cylinder",

"feature": "gear_bore"

}

},

"part_b": {

"part_id": "BSA-1",

"reference_entity": {

"entity_type": "face",

"surface_type": "cylinder",

"feature": "shaft_diameter_14mm"

}

},

"constraint_params": {

"keyed_connection": true,

"key_ref": "BSA-2"

}

}

],

"part_features": {

"BSA-1": {

"part_name": "Main Shaft with Roller",

"faces": [

{

"face_id": 1,

"surface_type": "cylinder",

"label": "shaft_journal",

"diameter": 10.0,

"length": 25.0

},

{

"face_id": 2,

"surface_type": "cylinder",

"label": "shaft_body",

"diameter": 14.0,

"length": 80.0

},

{

"face_id": 3,

"surface_type": "plane",

"label": "keyway",

"feature_type": "rectangular_through_slot",

"width": 5.0,

"depth": 2.5

},

{

"face_id": 4,

"surface_type": "cylinder",

"label": "circlip_groove",

"feature_type": "o_ring_groove",

"groove_width": 1.2

}

]

}

},

"views": {

"isometric": {

"scale": "1:2",

"position": "upper_left"

},

"section_A-A": {

"scale": "2:3",

"cutting_plane": "horizontal_centerline"

}

},

"labelling_info": {

"labeller_id": "annotator_imperial_001",

"labelling_date": "2026-01-26T13:07:56Z",

"quality_score": 0.95,

"notes": "Assembly includes standard components (circlips, bearings, gear) with custom machined parts (shaft, roller, bush, flange)"

}

}

# 8. Complexity Challenges and Labelling Design Decisions

CAD data labelling involves numerous complex decisions that significantly impact downstream ML model performance. This section addresses critical challenges and recommends best practices based on state-of-the-art research (AutoMate, BRepNet, Hierarchical CADNet).

## 8.1 Coordinate system representation: why NOT to use raw XYZ

Critical Decision: Assembly mates and feature positions should NOT be labelled using absolute XYZ coordinates and Euler angles. This approach is fundamentally flawed for ML applications:

- Fragility to transformations: If a part is moved or rotated in the assembly, all XYZ-based labels become invalid. Models trained on absolute coordinates fail to generalise to the same part in different positions.

- Scale dependency: XYZ coordinates are scale-dependent. A hole at (10, 20, 5) in a 100mm part is semantically different from (10, 20, 5) in a 1000mm part.

- Origin inconsistency: Different CAD systems and designers place the part origin differently. Raw XYZ values are meaningless without knowing the origin convention.

### Recommended approach: Topological references + Mating coordinate frames (MCFs)

Following the AutoMate dataset methodology (Jones et al., 2021), mates and positions should be defined relative to B-Rep topological entities:

- Reference by entity ID: Instead of 'hole centre at (15, 20, 7.5)', use 'hole_1 is centred on face_id=23 (cylindrical face)'. The face ID is stable regardless of part position.

- Mating coordinate frames (MCFs): Define a local coordinate frame attached to a topological entity. This frame moves with the part, maintaining semantic consistency.

## 8.2 Assembly mate type taxonomy

Based on the AutoMate dataset (541,635 unique mates), assembly mates should be classified into standard kinematic joint types:

| Mate Type | DOF | Description | Reference Entities |
| --- | --- | --- | --- |
| Fastened | 0 | Rigid connection, no relative motion | Face-Face or Edge-Edge |
| Revolute | 1 | Rotation about single axis (hinge) | Axis (cylinder/cone) + Face |
| Cylindrical | 2 | Rotation + translation along axis | Cylinder-Cylinder |

Table 5: Assembly mate types with degrees of freedom

# 9. Data Preprocessing and Simplification

Raw CAD data requires extensive preprocessing before ML model training. This section details each simplification step with technical specifications.

## 9.1 Geometric simplifications

- Defeaturing (Feature Suppression): Small features that don't contribute to the learning objective should be removed. Identify faces with area < threshold (typically 1mm² or 0.1% of total surface area). Use CAD kernel operations to remove small fillets, chamfers, logos, and text.

- Scale normalisation: Models span orders of magnitude in size. Compute bounding box diagonal d = √(L² + W² + H²). Scale model by 1/d to fit in unit sphere. Record scale factor for inverse transformation.

- Orientation alignment: Compute principal component analysis (PCA) of vertex positions. Align principal axes with coordinate axes. Reduces the space of possible transformations the model must learn.

## 9.2 Feature extraction for ML representations

- Voxel grid generation: Convert solid to 3D occupancy grid using ray-casting. Standard resolutions: 32³, 64³, 128³. Use case: 3D CNNs (FeatureNet, VoxNet).

- Point cloud sampling: Sample points uniformly from mesh surface using Poisson disk sampling. Standard sizes: 1024, 2048, 4096 points. Use case: PointNet, PointNet++, DGCNN.

- Face adjacency graph (AAG) Construction: Build graph where nodes=faces, edges=shared boundaries. For each node, store: face_id, surface_type, area, centroid, normal vector. Use case: Graph Neural Networks (Hierarchical CADNet, BRepNet, AAGNet).

# 10. CAD Labelling Approaches and Industry Technologies

Data labeling for Computer-Aided Design (CAD) involves annotating, classifying, and structuring 2D or 3D design data to make it machine-readable for AI, engineering, or GIS applications. This process is crucial for automating design reviews, enabling Design for Manufacturability (DFM), generating G-code, and improving data loss prevention (DLP).

## 10.1 Key approaches and technologies

- Automatic feature recognition (AFR): Advanced systems can analyse 2D/3D engineering drawings to automatically identify and label key features like holes, slots, or dimensions. This aligns with Chen et al.'s methodology of identifying specific geometrical characteristics.

- Metadata tagging: Tools like Fortra CAD Classifier allow users to add classification metadata (e.g., 'Confidential', 'Proprietary') directly into CAD files (DWG, DXF) to enforce security policies.

- Object data mapping (AutoCAD Map 3D/Civil 3D): CAD objects can be linked to external data sources (like SHP files), and these data fields can be automatically converted into text labels.

- Annotation templates: Creating specialised annotation blocks that pull attribute data from CAD objects allows for dynamic labelling that updates if the original data changes.

## 10.2 Chen et al.'s revolutionary approach (2024)

Chen et al. introduced a high-efficiency technology for recognising engineering drawings, automatically labelling key features, and converting unstructured data into structured data. Their methodology differs from traditional neural network approaches:

- Import DXF file (unstructured data)

- Convert to object entities (m lines, n circles/arcs, p polylines)

- Identify specific geometrical characteristics (anchor points)

- Apply domain knowledge (SPC rules, company/customer preferences)

- Inference key entities and auto-label

- Extract necessary entities (m' lines, n' circles/arcs, p' polylines)

- Export structured data for subsequent programs

## 10.3 Common tools and plugins

- Autodesk AutoCAD/Map 3D/Civil 3D: Contains built-in tools for labelling, annotations, and FDO (Feature Data Objects).

- Fortra CAD classifier: Integrates with AutoCAD to apply classification and visual labels.

- SECUDE HALOCAD: Used for securing and classifying CAD files with Microsoft Information Protection (MIP).

- AI-Powered annotation platforms: Specialised platforms for training computer vision models to recognise engineering drawing components.

## 10.4 Best practices for CAD labelling

- Consistency: Use predefined annotation templates to ensure all labels are consistent.

- Automate when possible: Utilise tools like MAPLABEL2ANN in Map 3D to convert GIS features to text automatically.

- Ensure data persistence: Use tools that embed data into the file format, ensuring labels remain even if the drawing is moved or shared.

- Verify accuracy: Implement quality assurance to verify that labels accurately reflect the underlying object properties.

# 11. Labeller Training Programme

CAD data labelling requires specialised domain knowledge. Labellers must understand engineering conventions, manufacturing processes, and CAD software interactions. This section details a comprehensive training programme.

## 11.1 Training curriculum

### Module 1: CAD Fundamentals

- Introduction to CAD and B-Rep: History of CAD, solid modelling concepts, B-Rep data structure

- Surface Types and Geometry: Analytic surfaces (plane, cylinder, cone, sphere, torus), free-form surfaces

- CAD File Formats: Native vs neutral formats, STEP structure, common conversion issues

- Manufacturing Processes Overview: Subtractive, additive, forming processes

### Module 2: Machining Feature Recognition

- Hole Features: Through-holes, blind holes, counterbores, countersinks, tapped holes

- Pocket and Slot Features: Rectangular, circular, T-slots, dovetails

- Transition Features: Fillets, rounds, chamfers

### Module 3: Quality Standards

- Inter-annotator agreement (Cohen's Kappa target: κ > 0.85)

- Gold standard maintenance and calibration

# 12. Project Scope, Timeline, and Scaling

| Phase | Duration | Volume | Team | Milestone |
| --- | --- | --- | --- | --- |
| Pilot | 6 weeks | 1,000 models | 2 labellers, 1 engineer, 1 QM | 90%+ accuracy |
| Scale-up | 4 months | 10,000-50,000 | 10 labellers, 2 engineers, 2 QM | IAA >0.85 |
| Production | --- | 100,000+/year | 20+ labellers, 3 engineers | Continuous |

Table 6: Project phasing and scaling plan

# 13. Future Scope and Strategic Opportunities

- Industry vertical expansion: Extend taxonomy to aerospace (structural brackets, turbine blades), medical devices (implants, surgical tools), automotive (powertrain, body panels), and consumer products.

- AI-Assisted labelling: Deploy pre-trained models (FeatureNet, Hierarchical CADNet) for automated pre-labelling. Expected efficiency gain: 3-5× throughput improvement.

- Online DFM feedback: Following Chen et al.'s extended applications, enable customers to upload drawings and receive immediate DFM examination feedback, online quotation, and supply chain notification.

- CNC programming integration: Export structured data in JSON format for compiling lathe cutting tool paths and robotic machining programs.

# 14. Implications and Strategic Considerations

## 14.1 Technical implications

- CAD data contains proprietary design IP requiring enterprise-grade security protocols aligned with Isidor's security and privacy capabilities.

- B-Rep to mesh/voxel conversion introduces geometric approximations. Labelling should occur on original B-Rep where possible.

- Large-scale CAD datasets (100,000+ models) require significant storage (10-50TB) and compute infrastructure.

## 14.2 Operational implications

- CAD labelling requires higher skill levels than typical annotation, impacting workforce recruitment and training costs.

- Taxonomy evolution based on client needs requires flexible guidelines and periodic labeller retraining.

# 15. Bibliography and references used

## 15.1 Academic papers

[1] Chen, T.C., Chang, T.H. and Lee, J.C. (2024) 'A revolutionary approach to automate labelling of CAD files', Journal of Physics: Conference Series, 2878, 012008. doi:10.1088/1742-6596/2878/1/012008

[2] Colligan, A.R., Robinson, T.R., Nolan, D.C., Hua, Y. and Cao, W. (2022) 'Hierarchical CADNet: Learning from B-Reps for Machining Feature Recognition', Computer-Aided Design, 147, 103226.

[3] Jones, B.E., Hildreth, D., Chen, D., Baran, I., Kim, V.G. and Schulz, A. (2021) 'AutoMate: A Dataset and Learning Approach for Automatic Mating of CAD Assemblies', ACM Transactions on Graphics, 40(6), Article 239.

[4] Lambourne, J.G., Willis, K.D.D., Jayaraman, P.K., Sanghi, A., Meltzer, P. and Shayani, H. (2021) 'BRepNet: A Topological Message Passing System for Solid Models', arXiv:2104.00706.

[5] Zhang, Z., Jaiswal, P., Rai, R. (2018) 'FeatureNet: Machining feature recognition based on 3D Convolution Neural Network', Computer-Aided Design, 101, pp.12-22.

[6] Jayaraman, P.K., Sanghi, A., Lambourne, J.G., Willis, K.D.D., Davies, T., Shayani, H. and Morris, N.J.W. (2021) 'UV-Net: Learning from Boundary Representations', CVPR 2021.

[7] Cao, W., Robinson, T., Hua, Y., Boussuge, F., Colligan, A.R. and Pan, W. (2020) 'Graph Representation of 3D CAD Models for Machining Feature Recognition With Deep Learning', ASME IDETC-CIE 2020.

[8] Shi, P., Qi, Q., Qin, Y., Scott, P.J. and Jiang, X. (2020) 'A novel learning-based feature recognition method using multiple sectional view representation', Journal of Intelligent Manufacturing, 31, pp.1291-1309.

## 15.2 Datasets

[9] MFCAD++ Dataset (2022) Queen's University Belfast. Available at: https://gitlab.com/qub_femg/machine-learning/mfcad2-dataset

[10] AutoMate Dataset (2021) Dryad. doi:10.5061/dryad.2547d7wvw

[11] MFCAD-VLM Dataset (2024) Zenodo. doi:10.5281/zenodo.14038050

[12] HybridCAD Dataset (2025) Zenodo. doi:10.5281/zenodo.14043179

## 15.3 Specifications

[13] ISO 10303-242:2020 Industrial automation systems and integration — Product data representation and exchange — Part 242: Application protocol: Model-based 3D engineering.

[14] ISO 14649 (STEP-NC) Industrial automation systems and integration — Physical device control — Data model for computerized numerical controllers.

## 15.4 Software and tools

[15] Open CASCADE Technology (OCCT) - Open source CAD kernel. https://dev.opencascade.org/

[16] PythonOCC - Python wrapper for OCCT. https://github.com/tpaviot/pythonocc

[17] Fortra CAD Classifier - Data classification for AutoCAD. https://www.fortra.com/

[18] SECUDE HALOCAD - CAD file security and classification. https://secude.com/

[19] Spatial Manager for AutoCAD - Advanced labelling tool. https://www.intl-spectrum.com/

## 15.5 Industry reports

[20] MarketsandMarkets (2022) 'Data Annotation and Labeling Market - Global Forecast to 2027'.

[21] Kraus, S., Jones, P., Kailer, N., Weinmann, A., Chaparro-Banegas, N. and Roig-Tierno, N. (2021) 'Digital Transformation: An overview of the current state of the art of research', SAGE Open, 11(3).

## 15.6 Patents

[22] Mahmood, S.A. et al. (2021) 'Extracting and analyzing information from engineering drawings', US Patent US2021089767A1.

## 15.7 References from Chen et al. (2024)

[23] Catalucci, S. et al. (2023) 'Smart optical coordinate and surface metrology', Measurement Science and Technology, 34, 012001.

[24] Jin, S. et al. (2021) 'Automatic 3D CAD Model and 2D Drawings Generation in Construction Engineering', Journal of Physics: Conference Series, 1827, 012115.

[25] Fan, X.G. et al. (2021) 'B-spline fitting of sunglasses lens contour based on adaptive sampling', Journal of Physics: Conference Series, 1914, 012041.

[26] Wong, F.S.Y. et al. (2014) 'Freeform surface measurement and characterisation using a toolmakers microscope', Journal of Physics: Conference Series, 483, 012004.

[27] Xu, Z. and Liu, Y.J. (2019) 'ABB robotic arm offline programming system', Journal of Physics: Conference Series, 1267, 012064.